ALU

ALU Specifications

- Operands: 4-bit (A, B)

- Opcode: 3-bit

- Operations: Arithmetic, Logic, Shift

- Type: Combinational

| Opcode | Operation |
| --- | --- |
| 000 | ADD |
| 001 | SUB |
| 010 | AND |
| 011 | OR |
| 100 | XOR |
| 101 | NOT |
| 110 | MUL |
| 111 | DIV |

POINTS TO REMEMBER

- MUL result is truncated to 4 bits

- Carry indicates multiplication overflow

- Division-by-zero handled safely.

- Overflow logic works only for signed operations

- Carry works for unsigned arithmetic

- While designing an ALU, careful attention must be paid to opcode definition, combinational logic implementation, overflow handling, and thorough verification through simulation to ensure correct and synthesizable hardware.

Overflow is related to signed number representation (2’s complement).

In signed arithmetic:

- MSB = sign bit

  - 0 → positive

  - 1 → negative

- Overflow happens when the sign of the result is incorrect.

Example (4-bit signed)

Range: −8 to +7

Unsigned numbers:

- No sign bit

- All bits represent magnitude

- Carry indicates result exceeds bit width

Example (4-bit unsigned)

Range: 0 to 15

Carry and Overflow are not used by the ALU itself; they are used by control logic / software to make decisions.

After the ALU computes:

- Result

- Carry

- Overflow

- (Zero, Sign)

These are stored in a status register (also called flag register).

Example flags:

- Carry (C)

- Zero (Z)

- Overflow (V)

- Sign (S)