Decoder

A decoder takes n input bits and produces 2ⁿ output lines, where only one output is active for any input combination.

Direct Decoder

A direct decoder implements each output using a single AND gate that includes all input bits (or their complements).

Example: Direct 3:8 Decoder

Characteristics

- Each output uses a 3-input AND gate

- Simple logic

- Easy to code

- Works well for small decoders

Limitations

- AND gate fan-in increases with input size

- Higher delay due to stacked transistors

- Increased power and area

- Poor scalability for large decoders

Pre-Decoded (Hierarchical) Decoder

A pre-decoded decoder splits the decoding into multiple smaller stages to reduce complexity.

Example: Pre-Decoded 3:8 Decoder

- First stage:

  - Decode (A, B) using a 2:4 decoder

  - Decode C using a 1:2 decoder

- Second stage:

  - Combine outputs using 2-input AND gates

Fan-in = number of inputs to a logic gate

| Decoder Type | AND Gate Fan-In |
| --- | --- |
| Direct 3:8 | 3 |
| Pre-decoded 3:8 | 2 |
| Large decoders | Reduced from N → 2 or 3 |

Even with two stages, pre-decoded is often faster because:

- No high fan-in gates

- Better transistor sizing

- Less RC delay

Pre-decoding does not reduce the number of AND operations; it reduces AND-gate fan-in, which lowers series transistor stacks, improving delay and power.

Large decoders are implemented using pre-decoding to reduce fan-in and improve timing and power.

Fan In

Fan-in of a logic gate is the number of inputs that the gate has.

Fan-in is not just a logic concept — it directly affects speed, power, and layout in VLSI.

AND Gate Fan-In at Transistor Level

CMOS AND Gate Implementation

An AND gate is usually built as:

- NAND gate + inverter

NAND gate pull-down network:

- NMOS transistors in series

- Number of series NMOS = fan-in

2-input AND  → fast

3-input AND  → slower

4-input AND  → much slower